# Technical Document: Mechanical Structure of Autonomous Mobile Robot

# 1. Overview

This document outlines the mechanical design and structure of a custom-built Autonomous Mobile Robot (AMR), primarily designed for indoor applications with modular sensing and computing capabilities. The robot frame consists of three 3D-printed plates made from CPE (copolyester) material, each with specific roles in supporting sensors, controllers, and drive mechanisms.

# 2. Material and Manufacturing

- Material: CPE (Copolyester)

- Manufacturing Method: FDM 3D Printing

- Plate Thickness: 3 mm (all three plates)

- Properties of CPE: High impact resistance, moderate flexibility, excellent dimensional stability under load and temperature.

# 3. Dimensions

| Plate | Length (mm) | Width (mm) | Thickness (mm) |
| --- | --- | --- | --- |
| Bottom Plate | 212 | 150 | 3 |
| Middle Plate | 212 | 150 | 3 |
| Top Plate | 150 | 150 | 3 |

# 4. Structural Design and Mounting

## 4.1 Bottom Plate

- Function: Mounts the motors, caster wheel, and power electronics.

- Reinforcement: Structural ribs and thickened motor mount sections improve load-bearing capability near motor fixation areas.

- Drive System:
  - 2 Rear Wheels: Driven by geared DC motors mounted on the bottom plate.
  - 1 Front Caster Wheel: Provides passive stability and maneuverability.

## 4.2 Middle Plate

- Function: Hosts computing and control units.

- Components Mounted:
  - NVIDIA Jetson Module
  - Camera System
  - Wiring Slots and Mounting Holes: Strategically designed for clean cable management and component mounting.

## 4.3 Top Plate

- Function: Sensor mounting.

- Components Mounted:
  - 2D LiDAR Sensor (centered and raised using a standoff mount)
  - Slots for airflow, wiring, and optional add-ons.

# 5. Load Handling and Deformation

The design was validated with finite element analysis (FEA) under a 2 kg payload, simulating the total weight from electronics and sensors.

- Max Deformation:

- Observation: The critical regions with minimal deformation were near the motor mount sections, proving structural reinforcement effectiveness.

# 6. Assembly and Modularity

All three plates are held together using M3 screws and standoffs.
The modular sandwich structure allows:

- Easy component replacement or upgrades.

- Quick access for maintenance.

- Light-weight yet rigid assembly, suitable for agile indoor navigation.

# 7. Summary

This AMR’s mechanical structure offers a lightweight, modular, and functional platform for autonomous tasks. The use of CPE and careful 3D printing design (with load-focused reinforcement) ensures structural durability, even with delicate components like Jetson and LiDAR mounted onboard.